# Phase 3 Channel and Mobility Model Reference

Tables and equations implemented in the Phase 3 simulation, transcribed from the project source documents 3GPP TR 36.777 V15.0.0 (36777-f00, parts 1 and 2) and 3GPP TR 38.901 (38901-j40). Every value below was read from the embedded equations of those documents; this reference exists so the implementation can be checked against the specifications independently. Known deliberate deviations are recorded separately in deviations_log.md.

Notation: fc is the carrier frequency in GHz unless stated otherwise; d2D and d3D are the horizontal and slant distances in metres; hUT and hBS are the UE and base station heights in metres above ground.

# 1. Applicability of the aerial overlay (TR 36.777 Annex B.1)

The aerial channel overlay applies between the z-boundary and 300 m above ground. Below the boundary, the standard terrestrial TR 38.901 model is used even for the aerial UE class. TR 36.777 Table B-1 Note 1 states the LOS probabilities assume the antenna heights listed.

| Scenario | z-boundary (m AGL) | Overlay range | gNB height assumption | Alt-1 fast fading |
| --- | --- | --- | --- | --- |
| UMa-AV | 22.5 | 22.5 - 300 m | 25 m | CDL-D based (Annex B.1.1) |
| RMa-AV | 10 | 10 - 300 m | 35 m | CDL-D based (Annex B.1.1) |
| UMi-AV | 22.5 | 22.5 - 300 m | 10 m | Reverse-UMa clause 7.5 (Annex B.1.1, final paragraph) |

# 2. LOS probability

## 2.1 Terrestrial band (TR 38.901 Table 7.4.2-1)

Implemented in tr36777LOSProbability.m for UEs at or below the z-boundary. d2D-out is taken as d2D (all UEs outdoor).

RMa:

PrLOS = 1 for d2D ≤ 10 m;  exp(−(d2D − 10)/1000) otherwise

UMi - Street Canyon:

PrLOS = 1 for d2D ≤ 18 m;  18/d2D + exp(−d2D/36)·(1 − 18/d2D) otherwise

UMa:

PrLOS = 1 for d2D ≤ 18 m; otherwise

PrLOS = [18/d2D + exp(−d2D/63)·(1 − 18/d2D)]·[1 + C′(hUT)·(5/4)·(d2D/100)3·exp(−d2D/150)]

C′(hUT) = 0 for hUT ≤ 13 m;  ((hUT − 13)/10)1.5 for 13 m < hUT ≤ 23 m

## 2.2 Aerial band (TR 36.777 Table B-1)

Common form for all three scenarios inside the overlay band, with d1 and p1 height-dependent (d2D in metres, Note 2):

PrLOS = 1 for d2D ≤ d1;  d1/d2D + exp(−d2D/p1)·(1 − d1/d2D) for d2D > d1

| Scenario | Height band | p1 | d1 |
| --- | --- | --- | --- |
| RMa-AV | 10 < hUT ≤ 40 m | max(15021·log₁₀(hUT) − 16053, 1000) | max(1350.8·log₁₀(hUT) − 1602, 18) |
| RMa-AV | 40 < hUT ≤ 300 m | PrLOS = 100% | - |
| UMa-AV | 22.5 < hUT ≤ 100 m | 4300·log₁₀(hUT) − 3800 | max(460·log₁₀(hUT) − 700, 18) |
| UMa-AV | 100 < hUT ≤ 300 m | PrLOS = 100% | - |
| UMi-AV | 22.5 < hUT ≤ 300 m | 233.98·log₁₀(hUT) − 0.95 | max(294.05·log₁₀(hUT) − 432.94, 18) |

# 3. Pathloss

## 3.1 Terrestrial band (TR 38.901 Table 7.4.1-1)

Evaluated in code by the toolbox function nrPathLoss with the scenario set per fork and EnvironmentHeight hE = 1 m for UMa/UMi; the formulas are reproduced here for checking. Breakpoint distances: d′BP = 4·(hBS − hE)·(hUT − hE)·fc·10⁹/c (UMa, UMi) and dBP = 2π·hBS·hUT·fc·10⁹/c (RMa), with c = 3×10⁸ m/s.

UMa LOS (10 m ≤ d2D ≤ d′BP: PL₁; d′BP ≤ d2D ≤ 5 km: PL₂):

PL1 = 28.0 + 22·log₁₀(d3D) + 20·log₁₀(fc)

PL2 = 28.0 + 40·log₁₀(d3D) + 20·log₁₀(fc) − 9·log₁₀((d′BP)2 + (hBS − hUT)2)

UMa NLOS:

PL = max(PLUMa-LOS, PL′);  PL′ = 13.54 + 39.08·log₁₀(d3D) + 20·log₁₀(fc) − 0.6·(hUT − 1.5)

RMa LOS (h = average building height, 5 m default; W = street width, 20 m default):

PL1 = 20·log₁₀(40π·d3D·fc/3) + min(0.03·h1.72, 10)·log₁₀(d3D) − min(0.044·h1.72, 14.77) + 0.002·log₁₀(h)·d3D

PL2 = PL1(dBP) + 40·log₁₀(d3D/dBP)

RMa NLOS:

PL = max(PLRMa-LOS, PL′);  PL′ = 161.04 − 7.1·log₁₀(W) + 7.5·log₁₀(h) − (24.37 − 3.7·(h/hBS)2)·log₁₀(hBS) + (43.42 − 3.1·log₁₀(hBS))·(log₁₀(d3D) − 3) + 20·log₁₀(fc) − (3.2·(log₁₀(11.75·hUT))2 − 4.97)

UMi - Street Canyon LOS (same distance split as UMa):

PL1 = 32.4 + 21·log₁₀(d3D) + 20·log₁₀(fc)

PL2 = 32.4 + 40·log₁₀(d3D) + 20·log₁₀(fc) − 9.5·log₁₀((d′BP)2 + (hBS − hUT)2)

UMi NLOS:

PL = max(PLUMi-LOS, PL′);  PL′ = 35.3·log₁₀(d3D) + 22.4 + 21.3·log₁₀(fc) − 0.3·(hUT − 1.5)

## 3.2 Aerial band (TR 36.777 Table B-2)

Implemented in tr36777AerialPathloss.m. FSPL denotes free-space pathloss (Note 3 of Table B-2): FSPL = 32.45 + 20·log₁₀(d3D) + 20·log₁₀(fc).

| Scenario / condition | Pathloss (dB) | Stated validity |
| --- | --- | --- |
| RMa-AV LOS | max(23.9 − 1.8·log₁₀(hUT), 20)·log₁₀(d3D) + 20·log₁₀(40π·fc/3) | 10 < hUT ≤ 300 m, d2D ≤ 10 km |
| RMa-AV NLOS | max(PLRMa-AV-LOS, −12 + (35 − 5.3·log₁₀(hUT))·log₁₀(d3D) + 20·log₁₀(40π·fc/3)) | 10 < hUT ≤ 300 m, d2D ≤ 10 km |
| UMa-AV LOS | 28.0 + 22·log₁₀(d3D) + 20·log₁₀(fc) | 22.5 < hUT ≤ 300 m, d2D ≤ 4 km (no breakpoint, Note 1) |
| UMa-AV NLOS | −17.5 + (46 − 7·log₁₀(hUT))·log₁₀(d3D) + 20·log₁₀(40π·fc/3) | 10 < hUT ≤ 100 m, d2D ≤ 4 km |
| UMi-AV LOS | max(FSPL, 30.9 + (22.25 − 0.5·log₁₀(hUT))·log₁₀(d3D) + 20·log₁₀(fc)) | 22.5 < hUT ≤ 300 m, d2D ≤ 4 km |
| UMi-AV NLOS | max(PLUMi-AV-LOS, 32.4 + (43.2 − 7.6·log₁₀(hUT))·log₁₀(d3D) + 20·log₁₀(fc)) | 22.5 < hUT ≤ 300 m, d2D ≤ 4 km |

# 4. Shadow fading (TR 36.777 Table B-3)

Lognormal, standard deviation in dB. Implemented in tr36777ShadowFadingStd.m; drawn once per link when enabled. Terrestrial band values follow TR 38.901 Table 7.4.1-1.

| Scenario / condition | Aerial band σ (dB) | Terrestrial band σ (dB) |
| --- | --- | --- |
| RMa-AV LOS | 4.2·exp(−0.0046·hUT) (10 < hUT ≤ 300) | 4 (before breakpoint) |
| RMa-AV NLOS | 6 (10 < hUT ≤ 40) | 8 |
| UMa-AV LOS | 4.64·exp(−0.0066·hUT) (22.5 < hUT ≤ 300) | 4 |
| UMa-AV NLOS | 6 (22.5 < hUT ≤ 100) | 6 |
| UMi-AV LOS | max(5·exp(−0.01·hUT), 2) (22.5 < hUT ≤ 300) | 4 |
| UMi-AV NLOS | 8 (22.5 < hUT ≤ 300) | 7.82 |

# 5. Aerial fast fading, Alternative 1 (TR 36.777 Annex B.1.1) - UMa-AV and RMa-AV

Implemented in buildAerialCDL.m as an nrCDLChannel custom profile (the built-in CDL-D profile does not expose per-cluster ZOD, needed for Step 5). The procedure follows Annex B.1.1 Steps 1-6.

## 5.1 Desired parameters (Tables B.1.1-1 and B.1.1-2)

| Scenario | ASA | ASD | ZSA | ZSD | K | DS |
| --- | --- | --- | --- | --- | --- | --- |
| RMa-AV LOS | 0.2° | 0.2° | 0.1° | 0.1° | 20 dB | 10 ns |
| RMa-AV NLOS | 0.5° | 0.5° | 0.2° | 0.2° | 10 dB | 30 ns |
| UMa-AV LOS | 0.5° | 0.5° | 0.1° | 0.1° | 20 dB | 10 ns |
| UMa-AV NLOS | 1° | 1° | 0.3° | 0.3° | 10 dB | 30 ns |

## 5.2 CDL-D base profile (TR 38.901 Table 7.7.1-4)

Cluster 1 comprises the specular LOS ray (−0.2 dB) plus a Laplacian part at the same delay and angles. Per-cluster spreads: cASD 5°, cASA 8°, cZSD 3°, cZSA 3°; XPR 11 dB.

| # | PAS | Norm. delay | Power (dB) | AOD (°) | AOA (°) | ZOD (°) | ZOA (°) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular | 0 | −0.2 | 0 | −180 | 98.5 | 81.5 |
| 1 | Laplacian | 0 | −13.5 | 0 | −180 | 98.5 | 81.5 |
| 2 | Laplacian | 0.035 | −18.8 | 89.2 | 89.2 | 85.5 | 86.9 |
| 3 | Laplacian | 0.612 | −21 | 89.2 | 89.2 | 85.5 | 86.9 |
| 4 | Laplacian | 1.363 | −22.8 | 89.2 | 89.2 | 85.5 | 86.9 |
| 5 | Laplacian | 1.405 | −17.9 | 13 | 163 | 97.5 | 79.4 |
| 6 | Laplacian | 1.804 | −20.1 | 13 | 163 | 97.5 | 79.4 |
| 7 | Laplacian | 2.596 | −21.9 | 13 | 163 | 97.5 | 79.4 |
| 8 | Laplacian | 1.775 | −22.9 | 34.6 | −137 | 98.5 | 78.2 |
| 9 | Laplacian | 4.042 | −27.8 | −64.5 | 74.5 | 88.4 | 73.6 |
| 10 | Laplacian | 7.937 | −23.6 | −32.9 | 127.7 | 91.3 | 78.3 |
| 11 | Laplacian | 9.424 | −24.8 | 52.6 | −119.6 | 103.8 | 87 |
| 12 | Laplacian | 9.708 | −30.0 | −132.1 | −9.1 | 80.3 | 70.6 |
| 13 | Laplacian | 12.525 | −27.7 | 77.2 | −83.8 | 86.5 | 72.9 |

## 5.3 Scaling procedure

K-factor scaling (TR 38.901 clause 7.7.6). Laplacian cluster powers are rescaled and the model K-factor is defined by:

Pn,scaled = Pn,model − (Kdesired − Kmodel)      (eq 7.7.6-1)

Kmodel = PLOS − 10·log₁₀(Σn 10P_{n,model/10})      (eq 7.7.6-2)

After power scaling the delays are re-normalised to unit RMS delay spread (clause 7.7.6, steps 1-2), then scaled to the desired spread (clause 7.7.3):

τn,scaled = τn,model · DSdesired      (eq 7.7-1)

Angle scaling (clause 7.7.5.1, LOS-preserving form eq 7.7-6). The scale factor s is computed per Annex A.5: the smallest x ≥ 0 such that AS(x) = ASdesired, where

AS(x) = (180/π)·√(−2·ln|[PLOS·ejxφLOSπ/180 + Σn Pn·ejxφnπ/180] / [PLOS + Σn Pn]|)      (eq A-6)

with angles wrapped to (−180°, 180°]. The scaled cluster angles are then φn,scaled = s·(φn,model − φLOS,model) + φLOS,desired, and the per-cluster ray spreads are multiplied by the same s (Annex B.1.1 Step 3 requires the scaling to act on ray angles). This solve reproduces the published Table 7.7.5.1-1 factors exactly (e.g. CDL-D desired ZOD spread 1° gives s = 0.4477).

## 5.4 ZOD offset (Annex B.1.1 Steps 5 and 6)

In LOS conditions an offset is added to the non-direct (Laplacian) cluster ZODs after angle scaling; the LOS ray is untouched. In NLOS conditions the offset is zero (Step 6). h is the average building height (20 m working assumption for UMa).

RMa-AV (eq B.1.1-1, ground reflection):   μZOD,offset = arctan((hBS + hUT)/d2D) + arctan((hUT − hBS)/d2D)

UMa-AV (eq B.1.1-2, rooftop reflection):   μZOD,offset = arctan((hBS + hUT − 2h)/d2D) + arctan((hUT − hBS)/d2D)

# 6. Terrestrial fast fading (below the z-boundary)

Links below the z-boundary use built-in CDL-D (LOS) or CDL-C (NLOS) scaled to the scenario median RMS delay spread, DS = 10μlgDS seconds, from TR 38.901 Table 7.5-6 evaluated at the carrier frequency (values from the provided 38901-j40 document, which carries the Release 19 updates):

| Scenario | mean lgDS, LOS | mean lgDS, NLOS |
| --- | --- | --- |
| UMa | −7.067 − 0.0794·log₁₀(fc) | −6.47 − 0.134·log₁₀(fc) |
| RMa | −7.49 | −7.43 |
| UMi | −0.18·log₁₀(1 + fc) − 7.28 | −0.22·log₁₀(1 + fc) − 6.87 |

Ray offset angles within a cluster (TR 38.901 Table 7.5-3, rms spread normalised to 1): ±0.0447, ±0.1413, ±0.2492, ±0.3715, ±0.5129, ±0.6797, ±0.8844, ±1.1481, ±1.5195, ±2.1551.

# 7. UMi-AV fast fading: reverse-UMa clause 7.5 model (D3.4)

TR 36.777 Annex B.1.1, final paragraph: for UMi-AV, “a fast fading model based on the ‘reverse’ UMa scenario is used where the base station is below the average rooftop height and the UE is well above rooftop. In this alternative, the fast fading model in Section 7.5 of [4] is reused with the angular spreads at the base station and UE interchanged.” Implemented in umi/buildUMiAVChannel.m: ASD↔ASA and ZSD↔ZSA are swapped after drawing; all other UMi parameters are reused.

## 7.1 Large-scale parameters (TR 38.901 Table 7.5-6 Part-1, UMi - Street Canyon)

Lognormal: X = 10μ + σ·N(0,1). Note 7: use fc = 2 GHz for fc below 2 GHz. Below, L = log₁₀(1 + fc).

| Parameter | LOS μ | LOS σ | NLOS μ | NLOS σ |
| --- | --- | --- | --- | --- |
| lgDS | −0.18·L − 7.28 | 0.39 | −0.22·L − 6.87 | 0.19·L + 0.22 |
| lgASD | −0.05·L + 1.21 | 0.08·L + 0.29 | −0.24·L + 1.54 | 0.10·L + 0.33 |
| lgASA | −0.07·L + 1.66 | 0.021·L + 0.26 | −0.07·L + 1.76 | 0.05·L + 0.27 |
| lgZSA | −0.11·L + 0.81 | −0.03·L + 0.29 | −0.03·L + 0.92 | −0.05·L + 0.35 |
| K (dB) | 9 | 5 | n/a | n/a |

Other UMi values used: delay scaling rτ = 3 (LOS) / 2.1 (NLOS); clusters N = 12 / 19, rays M = 20; cluster spreads cASD 3°/10°, cASA 17°/22°, cZSA 7°/7°; per-cluster shadowing ζ = 3 dB; XPR μ = 9 / 8 dB.

ZSD and ZOD offset (TR 38.901 Table 7.5-8, UMi):

LOS:   μlgZSD = max(−0.21, −14.8·(d2D/1000) + 0.01·|hUT − hBS| + 0.83),  σ = 0.35,  μoffset,ZOD = 0

NLOS:  μlgZSD = max(−0.5, −3.1·(d2D/1000) + 0.01·max(hUT − hBS, 0) + 0.2),  σ = 0.35

NLOS:  μoffset,ZOD = −10−1.5·log₁₀(max(10, d₂D)) + 3.3

## 7.2 Cluster generation (TR 38.901 clause 7.5, steps 5-7 as implemented)

Delays:   τ′n = −rτ·DS·ln(Xn),  Xn ~ U(0,1); normalise, sort      (eq 7.5-1/-2)

LOS delay scaling:   Cτ = 0.7705 − 0.0433·K + 0.0002·K2 + 0.000017·K3;  τnLOS = τn/Cτ      (eq 7.5-3/-4)

Powers:   P′n = exp(−τn·(rτ − 1)/(rτ·DS))·10−Z_{n/10},  Zn ~ N(0, ζ2); normalise      (eq 7.5-5/-6)

LOS:   P1,LOS = KR/(KR + 1);  Pn → Pn/(KR + 1)      (eq 7.5-7/-8)

Clusters more than 25 dB below the strongest are removed. Azimuth angles by inverse Gaussian (eq 7.5-9 to -12), zenith by inverse Laplacian (eq 7.5-14 to -16):

φ′n = 2·(ASA/1.4)·√(−ln(Pn/max P)) / Cφ      (eq 7.5-9)

θ′n = −ZSA·ln(Pn/max P) / Cθ      (eq 7.5-14)

LOS case:   Cφ → Cφ·(1.1035 − 0.028·K − 0.002·K2 + 0.0001·K3)

LOS case:   Cθ → Cθ·(1.3086 + 0.0339·K − 0.0077·K2 + 0.0002·K3)

Random sign Xn ∈ {−1, 1} and perturbation Yn ~ N(0, (AS/7)²) are applied; in LOS the first cluster is forced onto the LOS direction (eq 7.5-12/-16). Scaling constants (Tables 7.5-2 and 7.5-4):

| N clusters | 8 | 10 | 11 | 12 | 14 | 15 | 16 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cφ (azimuth) | 1.018 | 1.090 | 1.123 | 1.146 | 1.190 | 1.211 | 1.226 | 1.273 | 1.289 |
| Cθ (zenith) | 0.889 | 0.957 | 1.031 | 1.104 | 1.1072 | 1.1088 | 1.1276 | 1.184 | 1.178 |

# 8. Mobility parameters (TR 36.777 Table A.2.1-1) and TS 36.331 measurement model

Implemented in handoverManager.m (tr36777Mobility). The measurement metric is uplink SRS SINR rather than DL RSRP; this substitution is recorded in the deviations log.

| Parameter | Value | Use in code |
| --- | --- | --- |
| MeasurementInterval | 10 ms | scan period (scanPeriod) |
| L1 filtering | linear average over 200 ms sliding window | computeAverageSINR over l1WindowSec |
| L3RRMCoefficient k | 1 | Fn = (1 − a)·Fn−1 + a·Mn, a = (1/2)k/4 |
| A3Offset | 2 dB | entering condition: Mneighbour > Mserving + 2 dB |
| TimeToTrigger | 160 ms | condition must hold continuously; per-neighbour timers |
| HOPreparationDelay | 50 ms | delay before logical switch (with execution delay) |
| HOExecutionDelay | 40 ms | delay before logical switch |
| RSRPError σ | 1.22 dB | Gaussian error on the decision chain only (seeded) |
| MTS (ping-pong) | 1 s | handover back to previous cell within 1 s counts as ping-pong |
| Qin/Qout, T310/N310 | −6/−8 dB, 1 s/1 | not implemented (no RLF modelling); logged |

# 9. Implementation map

| Model element | Specification source | Implementation |
| --- | --- | --- |
| LOS probability (terrestrial + aerial) | 38.901 Table 7.4.2-1; 36.777 Table B-1 | core/tr36777LOSProbability.m |
| Aerial pathloss | 36.777 Table B-2 | core/tr36777AerialPathloss.m |
| Terrestrial pathloss | 38.901 Table 7.4.1-1 | nrPathLoss via core/tr36777ChannelModel.m |
| Shadow fading | 36.777 Table B-3 | core/tr36777ShadowFadingStd.m |
| Aerial CDL (UMa-AV, RMa-AV) | 36.777 Annex B.1.1; 38.901 7.7.1/7.7.3/7.7.5.1/7.7.6, A.5 | core/buildAerialCDL.m |
| Terrestrial CDL | 38.901 Tables 7.7.1-3/-4, 7.5-6 | core/createScenarioChannels.m |
| UMi-AV reverse UMa | 36.777 Annex B.1.1 (final para); 38.901 clause 7.5, Tables 7.5-6/-8 | umi/buildUMiAVChannel.m |
| Handover chain | 36.777 Table A.2.1-1; TS 36.331 5.5.3/5.5.4 | core/handoverManager.m |
| Windowed features / CSV | project D3.1 definition | core/extractWindowedFeatures.m, writeFeatureCSV.m |

Self-check: core/tr36777SelfTest.m asserts the implementation against hand-computed values of the equations above and validates the Annex A.5 solve against Table 7.7.5.1-1. Deliberate deviations and simplifications are recorded in deviations_log.md.